SR9900 驱动说明

一：SR9900点灯说明

1，寄存器有关位定义说明：

index ：0x5700； reg 0x0020	LED_SEL_CFG

The SR9900 supports customizable LED operation modes via OCP register offset 0x0020h ,Table 10 describes the different LED actions.

Table 1.   LED Select

| Bit | Symbol | RW | Description |
| --- | --- | --- | --- |
| 7:4 | LEDSEL1 | RW | LED Select for PINLED1 |
| 3:0 | LEDSEL0 | RW | LED Select for PINLED0 |

Configure OCP register offset 0x0020h to support your own LED signals. For example, if the value for LED_SEL_CFG is 0xA1h (10100001b), the LED actions are:

- LED 0: On only in 10M mode, with no blinking during TX/RX

- LED 1: On only in 100M mode, with TX/RX blinking

Table 1.   Customized LEDs

| Bit | LINK |  | ACT |
| --- | --- | --- | --- |
| Speed | Link 10M | Link 100M | - |
| LED 0 | Bit 0 | Bit 1 | Bit 3 |
| LED 1 | Bit 4 | Bit 5 | Bit 7 |

Table 3.   LED Feature Control-1

| Feature Control | Bit12 | Bit13 | Bit14 | Bit15 |
| --- | --- | --- | --- | --- |
| 0 | LED0 Low Active | LED1 Low Active | RSVD | Indicates Option 1 of Table 5 is selected |
| 1 | LED0 High Active | LED1 High Active | RSVD | Indicates Option 2 of Table 5 is selected |

Table 4.   LED Feature Control-2

| LED Pin | ACT=0 | ACT=1 |
| --- | --- | --- |
| LINK=0 | Floating | All Speed ACT |
| LINK>0 | Selected Speed LINK | Option 1 (see Table 5): Selected Speed LINK+ Selected Speed ACT Option 2 (see Table 5): Selected Speed LINK+ All Speed ACT |

Note 1: ACT means blinking TX and RX. LINK indicates Link 10M and Link 100M.

Note 2: There is a special mode: LED OFF Mode-> Set all bits to 0.

Table 5.   LED Option 1 & Option 2 Settings

| Link Bit |  | Active Bit | Description |  |  |
| --- | --- | --- | --- | --- | --- |
| 10 | 100 |  | Link | Option 1 LED Activity | Option 2 LED Activity |
| 0 | 0 | 0 | LED Off |  |  |
| 0 | 0 | 1 | - | Act10+Act100 | Act10+Act100 |
| 0 | 1 | 0 | Link100 | - | - |
| 0 | 1 | 1 | Link100 | Act100 | Act10+Act100 |
| 1 | 0 | 0 | Link10 | - | - |
| 1 | 0 | 1 | Link10 | Act10 | Act10+Act100 |
| 1 | 1 | 0 | Link10+Link100 | - | - |
| 1 | 1 | 1 | Link10+Link100 | Act10+Act100 | Act10+Act100 |

Note:

Act10 = LED blinking when Ethernet packets transmitted/received at 10Mbps.

Act100 = LED blinking when Ethernet packets transmitted/received at 100Mbps.

Link10 = LED lit when Ethernet connection established at 10Mbps.

Link100 = LED lit when Ethernet connection established at 100Mbps.

2, SR9900驱动灯的实现

在驱动中，有关灯的设置在函数set_led_select()来实现。具体如下：

ocp_data = ocp_read_word(tp, SR9900_SPECWAY_REG_INDEX, SR9900_SPECWAY_REG_LEDSEL);

ocp_data &= 0x0f00;

ocp_data |= 0x0038;

ocp_write_word(tp, SR9900_SPECWAY_REG_INDEX, SR9900_SPECWAY_REG_LEDSEL, ocp_data);

这里降等LED0 设置为了ACT, LED1 设置为了LINK STATUS，也就是LED0将会在数据传输的时候闪烁（注意网卡本身的ARP等任何报文都会闪烁）。LED1表明状态，网络连接上线为亮（一般是表征插入网线亮灯，否则灭灯）。

另外这里设置的是LED0,LED1都是低电位有效。所以需要与外部硬件电路匹配。

需要设置其他的灯的状态，直接修改其中的赋值两行就可以了。

三：在Linux系统平台建议客户直接用USB-CDC-ECM模式驱动（Linux3.14以及跟高版本的内核直接默认支持CDC类网卡设备（如果集成商有删减，在官方内核下载，添加进去即可使用））

也就是这个驱动：CDC Ethernet support (smart devices such as cable modems)

但是针对Linux早期版本CDC驱动无法使SR9900正常驱动工作的系统（如Linux3.10），需要做如下工作：

驱动加载具体选择：

- 修改linuxconfig ：make menuconfig

需要将linux内核驱动中的有关CDC_ETHER的驱动选择项目全部去掉，具体如下：

进入：Device Drivers   Network device dirver  USB Network Adapters

选择”Multi-purpose USB Networking Framework”

去掉”Sharp Zaurus (stock ROMs) and compatible”

去掉”CDC Ethernet support (smart devices such as cable modems”

必须按照步骤选择，否则最后一项CDC Ethernet support是不能去掉的。

- 启用不带CDC驱动的linux内核之后。需要将我们的SR9900的驱动也安装到drivers/net/usb/目录，请写入相应的Makefile和Kconfig，并且编译时选择模块加载。

- 安装驱动：

采用去掉CDC_ETHER并且安装SR9900的新的内核，然后执行：

modprobe mii

modprobe sr9900 （或者是编译或ko文件通过insmod sr9900.ko加载）

至此。SR9900驱动安装完毕。

PS：cdc_ether为linux提供的标准类通信设备类驱动（可以不判定VID/PID）；但是仅仅实现了RX/TX；针对IOCTL或者其他FEATURE类功能一概都没有实现。

三：真的使用我司提供的SR9900驱动，可以 使用SR9900 AUTOMAC功能说明

1， 驱动最开始有一个宏：SR9900_AUTOMAC，打开此宏定义，将启用AUTOMAC功能：可以外部省掉EEPROM，一次生成MAC地址之后，只要机器不刷机，永久不变化。

2， AUTOMAC注意点：

a，#define	SR9900_MAC_FILE	"/home/SR9900_MAC_ADDR"

这个地址需要根据客户实际情况填写，系统能读能写的一个文件地址并且掉电数据不消失的一个地址即可。

b，需要将此驱动编译成ko 模块方式来加载；不能编译进入内核模式（选择【*】）

c，此驱动的加载（模块加载）在init.rc文件中，需要放在文件系统挂载之后。